VoIP.ms SOFTPHONE iPhone Setup Guide Dedicated subaccount • inbound DID routing • caller ID • voicemail • testing

Purpose Use this guide when installing the official VoIP.ms Softphone on an iPhone. The procedure below is based on a configuration that successfully registered, placed outbound calls, and received inbound calls.

# Known-good settings

| SETTING | USE THIS VALUE |
| --- | --- |
| Device account | A new, dedicated VoIP.ms SIP subaccount for this phone |
| Username | Full SIP subaccount username, for example 123456_mobile |
| Auth User Name | The same full SIP subaccount username |
| Password | That subaccount’s SIP password — not the portal password |
| Server / POP | The selected VoIP.ms server; match the DID’s POP |
| Transport Protocol | UDP |
| NAT Traversal | Off, unless troubleshooting proves it is needed |
| VoiceMail Number | *97 for the mailbox associated with the subaccount |
| Inbound route | Route the DID to this subaccount or to a ring group containing it |

Security Never send or screenshot a SIP password, QR provisioning code, API credential, or recovery code. Avoid posting complete SIP usernames publicly. Use a strong, unique SIP password for every phone.

# 1. Gather the information before touching the phone

Record the non-secret information below. Keep passwords in a password manager rather than in this document.

☐ Person / phone name: ________________________________________________

☐ DID telephone number: ______________________________________________

☐ Subaccount label: ___________________________________________________

☐ Full SIP username: __________________________________________________

☐ VoIP.ms server / POP: _______________________________________________

☐ Outbound caller ID DID: _____________________________________________

☐ Voicemail mailbox: __________________________________________________

# 2. Create a dedicated VoIP.ms subaccount

Sign in to the VoIP.ms customer portal.

Open Sub Accounts → Create Sub Account.

Create one unique subaccount for this phone. Use a descriptive label such as mobile-alan or office-mobile-2.

Choose SIP as the protocol and set a strong, unique SIP password.

Select or record the intended VoIP.ms server / POP. The app and DID must use the matching server.

If offered, set the permitted outbound caller ID to the DID this user should present.

Save the subaccount and note the full SIP username, normally the main account number plus an underscore and the subaccount name.

Do not use the DID as the login A DID is a telephone number and inbound destination. It is not a SIP username. Also do not use the portal email address or portal password in the softphone.

# 3. Install and configure the iPhone app

Install VoIP.ms Softphone: Call & Text from Apple’s App Store.

Open the app and add a VoIP.ms account.

Enter the full SIP subaccount username, the subaccount SIP password, and the selected VoIP.ms server / POP.

Open Advanced Settings and enter the same complete SIP subaccount username in Auth User Name.

Set Transport Protocol to UDP. Leave NAT Traversal Off for the initial setup.

Set VoiceMail Number to *97 if this subaccount is associated with its voicemail mailbox.

Save the account and return to the dialer.

## Confirm registration before continuing

☐ The account label should no longer appear red on the dialer.

☐ In the VoIP.ms portal, the new subaccount should show Registered.

☐ If it remains red or says Unauthorized, stop and use the troubleshooting section before changing DID routing.

# 4. Route the DID for inbound calls

Successful outbound calling proves that the app authenticated. It does not automatically route incoming calls to the app.

In the VoIP.ms portal, open DID Numbers → Manage DID(s).

Edit the DID that should ring this iPhone.

Set Routing to SIP/IAX2 and select the new softphone subaccount.

Confirm that the DID POP/server matches the server configured in the app.

Save, wait about one minute, then call the DID from an outside phone.

Known symptom Outbound works, but inbound gives a fast busy for about three seconds and disconnects: the DID is usually still routed to Main Account, another subaccount, or an unregistered destination. Routing it to the new softphone subaccount fixes this pattern.

## If several phones should ring

Do not point the DID directly to only one device. Create a VoIP.ms Ring Group containing the desired SIP subaccounts, then route the DID to that ring group.

# 5. Configure outbound caller ID

Open Sub Accounts → Manage Sub Accounts and edit the phone’s subaccount.

Set CallerID Number to an eligible DID owned or verified on the account.

Save, place an outbound call to a separate phone, and confirm that the intended number appears.

Caller ID is separate from DID routing Inbound routing determines where calls ring. Outbound caller ID determines which number recipients see. Configure and test both.

# 6. Configure voicemail

Open DID Numbers → Voicemail → Manage Voicemail Accounts.

Create or edit the intended mailbox and set its numeric voicemail PIN. There is no universal default PIN.

Associate the mailbox with the DID/subaccount as required by the account’s voicemail design.

In the softphone’s Advanced Settings, use *97 for direct access to the associated mailbox.

Using *98 normally opens general voicemail access and may ask for both the mailbox number and PIN.

# 7. Verify emergency-service configuration

VoIP.ms E911 is configured under DID Numbers → Emergency Services. Select an E911-registered DID for any device that may place emergency calls. Do not place an unannounced 911 test call.

# 8. Complete the acceptance tests

Do not hand the phone to its user until every relevant test below passes.

☐ Registration: account is not red in the app and the portal shows the subaccount Registered.

☐ Echo test: call 4443 and confirm microphone and speaker audio.

☐ Outbound call: call an outside number and confirm clear two-way audio.

☐ Caller ID: confirm the correct business DID appears on the receiving phone.

☐ Inbound call: call the DID from an outside carrier and answer in the app.

☐ Background test: lock the iPhone for at least 30 minutes, then call it again.

☐ Overnight test: verify an inbound call after the phone has been idle overnight.

☐ Network-change test: switch between Wi-Fi and cellular data and test inbound and outbound calls.

☐ Voicemail: leave a message, verify the indicator, dial *97, and retrieve the message.

☐ SMS: if enabled for the DID, send and receive a test text and confirm notifications.

☐ Restart test: restart the iPhone and verify that registration and inbound calling recover.

# Troubleshooting reference

| SYMPTOM | LIKELY CAUSE | WHAT TO CHECK |
| --- | --- | --- |
| Red account name / Unauthorized | Authentication failed | Use the full subaccount username in both Username and Auth User Name. Re-enter the subaccount SIP password. Confirm the server. Do not use the DID, email address, or portal password. |
| Outbound works; inbound fast-busies for about 3 seconds | DID routing points elsewhere | DID Numbers → Manage DID(s): route the DID to the new subaccount or its ring group. Match the DID POP to the app server. |
| Voicemail asks for mailbox and password | The app is using *98 | Use *97 for the mailbox associated with the subaccount. Reset the mailbox PIN in Manage Voicemail Accounts if necessary. |
| Potentially Unauthorized Call Blocked email | VoIP.ms anti-fraud check after an IP/network change | Use the email’s authorization control, wait for the account to update, and contact VoIP.ms support if it repeats or does not clear. |
| One-way or no audio | Network, VPN, or transport issue | Turn off VPN temporarily, confirm UDP, retry on both Wi-Fi and cellular, and verify iPhone microphone permissions. |
| Works while open but misses calls later | Background/push registration or network transition | Check iPhone notification/background permissions, reopen the app, confirm the subaccount remains registered, and repeat the locked-phone and Wi-Fi/cellular tests. |

# Reusable handoff checklist

Use this page as the short checklist for each additional installation.

☐ Create a unique SIP subaccount for the phone.

☐ Record the full subaccount username and selected VoIP.ms server / POP.

☐ Install the official VoIP.ms Softphone on the iPhone.

☐ Enter the full username, subaccount SIP password, and matching server.

☐ Set Advanced Settings → Auth User Name to the same full username.

☐ Use UDP; start with NAT Traversal Off.

☐ Set VoiceMail Number to *97 when an associated mailbox is configured.

☐ Confirm the app is not red and the portal shows Registered.

☐ Route the DID to the softphone subaccount or appropriate ring group.

☐ Set and verify outbound caller ID on the subaccount.

☐ Confirm voicemail PIN/mailbox behavior.

☐ Verify E911 configuration for the DID used by the phone.

☐ Complete outbound, inbound, caller-ID, two-way-audio, locked-phone, network-change, voicemail, and restart tests.

☐ Give the user basic instructions and store credentials securely.

Final rule Registration, inbound routing, outbound caller ID, voicemail, and E911 are separate pieces. A successful outbound call does not prove that inbound routing, voicemail, or emergency-service configuration is correct.