Polycom Phone Setup for VoIP.ms

Single-phone configuration guide using a VoIP.ms sub-account

Use this guide after factory-resetting a Polycom/Poly desk phone. It is written for one phone, but repeat the same process for each of the three phones using a different VoIP.ms sub-account for each phone.

# What You Need Before Starting

The phone plugged into power/PoE and Ethernet on the same network as your computer.

The phone IP address.

The Polycom admin password. Factory default is usually 456.

Your VoIP.ms SIP username or sub-account username, for example 123456_subaccount.

Your VoIP.ms SIP password for that sub-account.

Your preferred VoIP.ms server, for example newyork.voip.ms, atlanta.voip.ms, chicago.voip.ms, etc.

The caller ID name you want displayed on outbound calls, preferably uppercase and 15 characters or fewer.

# Recommended Plan for Three Phones

Create one VoIP.ms sub-account per phone.

Name the sub-accounts clearly, such as frontdesk, office, and backoffice.

Configure one phone first and test it before repeating the process on the other two.

After the number port completes, route the DID in VoIP.ms to a ring group, IVR, or the specific sub-account you want to ring.

# Fill This In First

| Field | Your Value |
| --- | --- |
| Phone location/name |  |
| Phone IP address |  |
| Polycom admin password | 456, unless changed |
| VoIP.ms SIP username / sub-account |  |
| VoIP.ms SIP password |  |
| VoIP.ms server |  |
| Outbound caller ID name |  |
| Line label on phone |  |

# Step 1: Get the Phone IP Address

Let the phone boot completely. This may take 1-2 minutes after a reset.

On many Polycom phones, press Home/Menu, then go to Settings > Status > Network > TCP/IP Parameters.

Write down the IPv4 address, usually something like 192.168.1.50.

# Step 2: Open the Web Configuration Utility

From a computer on the same network, open a browser.

Go to https://PHONE-IP-ADDRESS/ such as https://192.168.1.50/.

If the browser shows a certificate warning, continue to the page. This is normal for local phone administration.

Log in as Admin. The factory default password is usually 456.

# Step 3: Set Time/NTP

This is not strictly required for SIP registration, but it keeps call logs and voicemail indicators sane.

Go to Simple Setup > Time Synchronization.

Set your time zone and NTP server if the page offers those fields.

Save the settings.

# Step 4: Configure Line 1 for VoIP.ms

Go to Settings > Lines, then select Line 1. Use the values below. Field names vary a little by Polycom model and firmware, so use the closest matching field name.

| Section | Field | Value to Enter |
| --- | --- | --- |
| Identification | Display Name | Outbound caller ID name, preferably uppercase, no special characters, 15 characters or fewer |
| Identification | Address | Your VoIP.ms SIP username/sub-account, e.g. 123456_frontdesk |
| Identification | Label | Short name shown beside the line button, e.g. Front Desk |
| Identification | SRTP | No / Disabled, unless you intentionally configured encryption |
| Identification | Server Auto Discovery | Disabled, if present |
| Authentication | Use Login Credentials | Disabled, if present |
| Authentication | Domain | Blank, unless your model requires the VoIP.ms server here |
| Authentication | User ID / Auth User ID | Your VoIP.ms SIP username/sub-account |
| Authentication | Password / Auth Password | Your VoIP.ms SIP password for this sub-account |
| Outbound Proxy | Address | Your selected VoIP.ms server, e.g. newyork.voip.ms |
| Outbound Proxy | Port | 5060 |
| Outbound Proxy | Transport | UDPOnly / UDP |
| Server 1 | Address | Your selected VoIP.ms server, same as above |
| Server 1 | Port | 5060 |
| Server 1 | Transport | UDPOnly / UDP |
| Server 1 | Expires | 300 seconds is a good starting point; some older guides show 60 |
| Server 1 | Subscription Expires | 300 seconds, or match Expires |
| Message Center | Callback Mode | Contact |
| Message Center | Callback Contact | *97 |

# Step 5: Save and Restart

Click Save/Submit at the bottom of the Line 1 page.

Restart the phone from Utilities > Restart Phone, or power-cycle it.

Wait for the phone to come fully back online.

# Step 6: Verify Registration

On the phone screen, the line icon should look registered/solid, not failed or empty.

In the VoIP.ms customer portal, check the sub-account registration status on the portal home or sub-account page.

Make an outbound test call from the phone.

If the DID has not ported yet, inbound calls to the old business number will still go to Nextiva until the port completes. That is expected.

# After the Port Completes

In VoIP.ms, route the ported DID to the correct destination: a single sub-account, ring group, IVR, voicemail, or call hunting rule.

Call the ported number from a cell phone and confirm the right phone or phones ring.

Test outbound caller ID by calling a cell phone.

Test voicemail, after-hours behavior, and all three phones before canceling Nextiva.

# Troubleshooting Quick Checks

| Problem | What to Check |
| --- | --- |
| Registration fails | Re-enter the SIP username and password carefully. Use the sub-account SIP password, not necessarily the website login password. |
| Cannot connect to server | Try a different VoIP.ms PoP server close to you, or temporarily test the server IP address to rule out DNS issues. |
| Phone registers then drops | Use a shorter registration expiry such as 300 seconds and enable NAT keep-alive/NAT mapping if present. |
| One-way audio | Check router/SIP ALG settings. If necessary, allow UDP 5060 and RTP range UDP 10000-20000. |
| Multiple phones on same network act oddly | Use separate sub-accounts. If needed, assign different local SIP ports per phone such as 5061, 5062, and 5063. |
| Voicemail button does not work | Set Message Center callback/contact to *97, then test again. |

# Notes

Do not cancel Nextiva until the port is confirmed complete and inbound calls are tested through VoIP.ms.

These instructions are based on VoIP.ms Polycom VVX 300/400-series and SoundPoint IP 331 documentation, plus the VoIP.ms registration troubleshooting guide.

Official references: https://wiki.voip.ms/article/Polycom_VVX_300,_400,_etc ; https://wiki.voip.ms/article/Polycom_Soundpoint_IP_331 ; https://wiki.voip.ms/article/Registration_issue