Polycom VVX 310 Post-Port Configuration Instructions

For reset Polycom phones after DIDs have ported to VoIP.ms

# Purpose

Use this after the DIDs have successfully ported and the Polycom VVX 310 phones have been factory reset. The goal is to register each phone to its own VoIP.ms SIP subaccount, route each ported DID to the correct phone, and test inbound/outbound calling before closing the project.

# Assumptions

The phone service provider is VoIP.ms.

There are three reset Polycom VVX 310 desk phones.

Each desk phone should have its own VoIP.ms SIP subaccount.

The ported desk-phone DIDs should route to those three Polycom phones.

The remaining ported numbers may route to forwarding, fax, voicemail, ring groups, or other destinations as needed.

# Information to Collect Before Touching the Phones

| Item | Value / Notes |
| --- | --- |
| VoIP.ms portal login |  |
| Preferred VoIP.ms POP/server | Example: newyork.voip.ms, atlanta.voip.ms, chicago.voip.ms |
| Desk phone 1 location/name |  |
| Desk phone 1 DID |  |
| Desk phone 1 SIP subaccount username | Example: 123456_frontdesk |
| Desk phone 1 SIP password |  |
| Desk phone 2 location/name |  |
| Desk phone 2 DID |  |
| Desk phone 2 SIP subaccount username |  |
| Desk phone 2 SIP password |  |
| Desk phone 3 location/name |  |
| Desk phone 3 DID |  |
| Desk phone 3 SIP subaccount username |  |
| Desk phone 3 SIP password |  |
| Outbound caller ID name | Prefer uppercase, 15 characters or fewer |
| Voicemail / after-hours destination |  |
| Fax destination | If applicable |

# High-Level Order of Work

Confirm the DIDs show active in VoIP.ms.

Create or verify one SIP subaccount per Polycom phone.

Route each desk-phone DID to the matching SIP subaccount.

Configure each reset Polycom VVX 310 through its web interface.

Verify SIP registration in both the phone and the VoIP.ms portal.

Test inbound calling, outbound calling, caller ID, voicemail/failover, and reboot persistence.

# Step 1: Confirm the Port Completed

Log in to VoIP.ms.

Go to DID Numbers > Manage DID(s).

Confirm each ported DID appears in the account and is active.

Do not cancel old carrier service until every important number has been tested through VoIP.ms.

# Step 2: Create or Verify SIP Subaccounts

In VoIP.ms, go to Sub Accounts.

Create one subaccount for each Polycom phone, named by location or desk.

Use a different SIP password for each phone if possible.

Set caller ID to the final DID for that phone.

Use the same VoIP.ms POP/server for the phone registration and the DID routing whenever possible.

| Phone | Recommended Subaccount Name | DID Routed to This Phone | Notes |
| --- | --- | --- | --- |
| Phone 1 | frontdesk or location-based name |  |  |
| Phone 2 | office or location-based name |  |  |
| Phone 3 | backoffice or location-based name |  |  |

# Step 3: Route Each Desk-Phone DID

In VoIP.ms, open DID Numbers > Manage DID(s).

Click Edit on the desk-phone DID.

Set Routing to SIP/IAX or SIP Account / Sub Account, depending on the VoIP.ms screen wording.

Choose the matching Polycom phone subaccount.

Confirm the POP/server matches the server used by the phone where VoIP.ms provides that option.

Save the DID settings.

Repeat for each Polycom desk-phone DID.

# Step 4: Find the Polycom Phone IP Address

Plug the phone into Ethernet and power/PoE.

Let the phone finish booting after the reset.

On the phone, press Home, then go to Settings > Status > Network > TCP/IP Parameters.

Write down the IPv4 address, for example 192.168.1.50.

# Step 5: Open the Polycom Web Configuration Utility

From a computer on the same network, open a browser.

Go to https://PHONE-IP-ADDRESS/ such as https://192.168.1.50/.

Accept the local certificate warning if shown.

Log in as Admin. The common post-reset admin password is 456 unless the phone has been changed.

# Step 6: Configure Line 1

Go to Settings > Lines > Line 1. Polycom firmware labels vary slightly; use the closest matching field names below.

| Polycom Field | Value to Enter |
| --- | --- |
| Line Active / Enable Line | Enabled |
| Display Name / Label | Desk name or DID, such as FRONT DESK or 215-xxx-xxxx |
| Address / User ID | VoIP.ms SIP subaccount username, such as 123456_frontdesk |
| Authentication User ID | Same VoIP.ms SIP subaccount username |
| Authentication Password | SIP password for that subaccount |
| Outbound Proxy / Server 1 Address / Registrar | Your VoIP.ms server, such as newyork.voip.ms |
| Server 1 Port | 5060 unless VoIP.ms or the network requires another supported SIP port |
| Transport | UDP unless you intentionally configured TCP/TLS |
| Register | Enabled / Yes |

Save or Submit the line settings.

Restart the phone if prompted, or use Utilities > Restart Phone.

Wait for the phone to come fully back online before testing.

# Step 7: Verify Registration

On the phone screen, Line 1 should show as registered/available, not failed.

In VoIP.ms, check the subaccount registration status. It should show the phone registered to the expected POP/server.

If registration fails, re-check username, SIP password, server name, and whether the phone is using the same subaccount assigned in VoIP.ms.

# Step 8: Test Calls

| Test | Expected Result | Pass / Fail / Notes |
| --- | --- | --- |
| Outbound call from each Polycom phone to a cell phone | Call completes and audio works both ways |  |
| Outbound caller ID | Correct business number/name appears |  |
| Inbound call to each desk-phone DID from a cell phone | Correct Polycom phone rings |  |
| Missed call / voicemail / failover | Expected destination handles unanswered calls |  |
| Reboot phone and test again | Phone re-registers automatically |  |
| Call between phones, if desired | Internal or external dialing works as intended |  |
| After-hours behavior | Calls route to the expected destination |  |
| Fax number, if applicable | Send and receive work |  |

# Troubleshooting Quick Checks

| Symptom | Likely Cause | What to Check |
| --- | --- | --- |
| Phone does not register | Wrong SIP username/password or server | Confirm subaccount username, SIP password, and VoIP.ms POP/server. |
| Inbound calls do not ring the phone | DID is routed to the wrong destination | Check DID Numbers > Manage DID(s) and route the DID to the matching subaccount. |
| Outbound works but inbound fails | DID routing or POP mismatch | Make sure DID routing points to the same subaccount/server the phone is registered to. |
| One-way audio | NAT/firewall issue | Try another supported VoIP.ms SIP port, confirm router SIP ALG is off if it causes issues, and test from the same network. |
| Phone loses settings after reboot | Old provisioning or zero-touch config may still be active | Check provisioning server settings and disable old Nextiva/Poly provisioning if present. |
| Wrong caller ID | Subaccount or DID caller ID setting is wrong | Check outbound caller ID on the VoIP.ms subaccount and DID settings. |

# Final Cutover Checklist

All three Polycom phones register to VoIP.ms.

Each desk-phone DID rings the correct phone.

Outbound caller ID is correct for each phone.

Two-way audio works on inbound and outbound calls.

Voicemail, failover, and after-hours behavior work.

The fax number works if one was ported.

Any forwarded numbers reach the right cell phones or destinations.

The phones keep working after a reboot.

Only after testing is complete should old carrier services or unused numbers be cancelled.

# Reference Links

VoIP.ms Polycom VVX setup: https://wiki.voip.ms/article/Polycom_VVX_300%2C_400%2C_etc

VoIP.ms Sub Accounts: https://wiki.voip.ms/article/Sub_Accounts

VoIP.ms Manage DID routing: https://wiki.voip.ms/article/Manage_DID

VoIP.ms registration troubleshooting: https://wiki.voip.ms/article/Registration_issue

Prepared for post-port Polycom configuration. Last updated: June 23, 2026.