VoIP.ms Fax Line Setup Checklist

Ported DID + Axacore FaxBridge + HP 4100 MFP

# 1. Confirm the Number

[ ] Confirm the Nextiva number has fully ported into VoIP.ms and appears as an active DID.

[ ] Do not buy an additional VoIP.ms fax number unless you want a second fax line.

[ ] Expect to pay the normal monthly VoIP.ms DID fee for the ported number.

[ ] Decide whether this number will be dedicated to fax only, not shared with voice calls.

# 2. Create a VoIP.ms Fax Device Login

[ ] Log in to the VoIP.ms portal.

[ ] Create a new sub-account for the fax device/FaxBridge.

[ ] Set a strong SIP password for the sub-account.

[ ] Choose the closest reliable VoIP.ms server/POP, such as newyork.voip.ms if local to the Northeast.

[ ] Route the ported DID to the fax sub-account.

[ ] If available on the DID route, enable or prefer T.38 fax handling.

# 3. Configure the Axacore FaxBridge

[ ] Connect FaxBridge Ethernet to the router/network.

[ ] Connect power and allow the FaxBridge to boot fully.

[ ] Open the FaxBridge admin/configuration interface.

[ ] Enter the VoIP.ms SIP server, for example: newyork.voip.ms.

[ ] Enter the VoIP.ms sub-account SIP username.

[ ] Enter the VoIP.ms sub-account SIP password.

[ ] Set preferred codec to G.711u / PCMU.

[ ] Disable compressed codecs such as G.729.

[ ] Enable T.38 if the FaxBridge supports it and VoIP.ms accepts it.

[ ] If T.38 does not work, use G.711 passthrough.

[ ] Confirm the FaxBridge shows registered/online in its interface and/or in VoIP.ms registration status.

Note: Some Axacore FaxBridge units are provider/cloud-managed appliances and may not behave like generic SIP ATAs. If the box does not expose SIP server/username/password settings, it may need Axacore service involvement or replacement with a standard ATA such as a Grandstream HT801/HT802.

# 4. Wire the HP 4100 MFP

[ ] Connect the HP 4100 fax LINE port to the FaxBridge RJ-11/FXS phone port.

[ ] Do not connect the HP fax port directly to an old phone wall jack unless that jack is known to be disconnected from legacy telco voltage.

[ ] If the HP has both LINE and EXT ports, use LINE for the FaxBridge connection.

[ ] Keep the phone cable short and direct while testing.

# 5. Configure the HP 4100 Fax Settings

[ ] Set the fax header/CSID to the ported fax number.

[ ] Set dialing mode to tone dialing.

[ ] Set answer mode to Auto if inbound fax receive is required.

[ ] Start with 2 to 4 rings before answer.

[ ] Set fax speed to 9600 or 14400 if the HP offers speed settings.

[ ] Avoid max/33600 speed for VoIP fax reliability.

[ ] Leave ECM enabled for the first test if T.38 is working.

[ ] If faxes fail partway through or retry repeatedly, disable ECM and test again.

# 6. Test Inbound Fax

[ ] Send a test fax from an outside fax source to the ported number.

[ ] Confirm the VoIP.ms DID routes the call to the fax sub-account.

[ ] Confirm the FaxBridge rings the HP 4100.

[ ] Confirm the HP answers and prints/receives the fax.

[ ] Check VoIP.ms call detail records for completion status and call duration.

[ ] Repeat with a multi-page fax to verify stability, not just ring/answer.

# 7. Test Outbound Fax

[ ] Send a one-page outbound fax from the HP 4100 to a known working fax number.

[ ] Confirm the recipient receives a clean page.

[ ] Send a multi-page outbound fax.

[ ] Confirm outbound caller ID presents the ported fax number.

[ ] Check VoIP.ms call detail records for successful outbound completion.

# 8. If Faxing Is Unreliable

[ ] Confirm only G.711u / PCMU and/or T.38 are being used.

[ ] Disable G.729 and other compressed codecs.

[ ] Try a lower fax speed on the HP, starting with 9600.

[ ] Toggle ECM off if using G.711 passthrough.

[ ] Try a different nearby VoIP.ms POP/server.

[ ] Check firewall/NAT behavior and make sure SIP/RTP traffic is not being mangled by SIP ALG.

[ ] If the Axacore box cannot register directly to VoIP.ms, replace it with a known-good generic ATA or use Axacore service.

# Quick Wiring Reference

HP 4100 fax LINE port -> FaxBridge RJ-11/FXS port -> Ethernet/router -> VoIP.ms sub-account -> ported DID

# Recommended Starting Values

| Setting | Starting Value |
| --- | --- |
| DID | Use the ported Nextiva number now active at VoIP.ms |
| VoIP.ms route | DID -> fax sub-account |
| SIP server | Closest reliable VoIP.ms POP, e.g. newyork.voip.ms |
| Codec | G.711u / PCMU |
| T.38 | Enable if supported end-to-end |
| HP fax speed | 9600 or 14400 |
| ECM | On for T.38 test; off if G.711 passthrough is flaky |