Grandstream HT801 → HP LaserJet 4345 MFP

Configuration and test sheet for fax service through VoIP.ms

Do this first: The fax DID must ring normally from an outside phone before inbound fax can work. A fast-busy signal means the number is not yet correctly active or inbound-routed at the provider. Fix that with the DID provider; it is not an HT801 setting.

# What this setup does

Internet / router  →  Grandstream HT801  →  HP 4345 MFP fax port  The HT801 supplies the analog phone line; the HP sends and receives faxes on it. This guide uses a dedicated VoIP.ms sub-account for the fax line.

# Before you begin

A working Internet connection at the HT801 location and one Ethernet cable.

An RJ-11 telephone cable from the HT801 PHONE port to the HP 4345 MFP FAX/LINE port — not the HP phone-extension port.

The VoIP.ms DID already assigned to this fax line, confirmed active, and a dedicated VoIP.ms sub-account.

The HT801 administrator password from its underside label or the password already set on the unit. Do not factory-reset it unless necessary.

A one-page test document and a known working receiving fax number.

# Write these down before touching the HT801

| Item | Your value |
| --- | --- |
| VoIP.ms POP / SIP server | ________________________________________ |
| VoIP.ms fax sub-account / SIP username | ________________________________________ |
| Sub-account SIP password | ________________________________________ |
| Fax DID (10 digits) | ________________________________________ |
| HT801 local IP address | ________________________________________ |
| HP fax name / header text | ________________________________________ |

# 1. Set up the VoIP.ms side

In the VoIP.ms portal, create a dedicated sub-account for this fax line. Record its full SIP username and password above. Do not use the DID itself as the SIP username.

Select one nearby VoIP.ms POP / server for that sub-account and keep it consistent. Use the hostname VoIP.ms provides (for example, a city-number server), not an IP address.

Route the fax DID to that sub-account: DID Numbers → Manage DIDs → select the DID → Routing → the fax sub-account. Save the change.

Confirm the DID can receive an ordinary outside call. If callers still get fast busy, stop here and have VoIP.ms verify the DID is active and inbound-routed.

Important: Do not enable call forwarding, IVR, or a second destination on this DID while testing. Keep the route direct to the fax sub-account.

# 2. Wire and reach the HT801

With power disconnected, connect the HT801 Internet port to your router or switch. Connect the HT801 PHONE port to the HP 4345 MFP FAX/LINE port using a normal RJ-11 cord. Then power on the HT801.

Wait about two minutes for the HT801 to obtain a network address.

To hear its IP address: plug a basic analog phone temporarily into the HT801 PHONE port, lift the handset, press *** when you hear dial tone, then press 02. Write down the announced IP address. Reconnect the HP afterward.

From a computer on the same network, open http://<HT801-IP-address> in a browser. Sign in as admin using the password on the bottom label or the password previously set. (On older firmware it may still be admin.)

If you cannot open the page: Make sure the computer and HT801 are on the same local network, use http rather than https first, and re-check the IP from the IVR. Avoid a reset until those checks are exhausted.

# 3. Enter the HT801 SIP settings

Open the FXS PORT page (the exact tab name can vary slightly by firmware). Set the following baseline values, then click Apply / Save and reboot if the page asks you to.

| Setting | Value | Why |
| --- | --- | --- |
| Account Active | Yes | Enables the FXS line. |
| Primary SIP Server | Your VoIP.ms POP hostname | The server selected for the fax sub-account. |
| SIP User ID | VoIP.ms fax sub-account username | Registration username. |
| Authenticate ID | Same VoIP.ms fax sub-account username | Authentication username. |
| Authenticate Password | Sub-account SIP password | Not the web-portal password. |
| Name / Display Name | Fax DID or “HP 4345 Fax” | Caller ID label only; it is not the login. |
| NAT Traversal | Keep-Alive | Usually the best home/office-router baseline. |
| SIP transport | UDP (baseline) | Use the provider’s required transport if it differs. |
| DTMF method | RFC2833 | Normal VoIP.ms-compatible setting. |
| SRTP | Disabled unless VoIP.ms explicitly requires it | Avoids an unnecessary variable while testing. |

# 4. Set the HT801 for fax reliability

Still on the FXS PORT page, find the fax and audio sections. Names may vary a little by HT801 firmware.

Fax Mode: set T.38 (Auto Detect). If there is an “Enable T.38” control, enable it.

If the page offers “Re-INVITE after fax tone detected,” enable it.

Preferred vocoder / codec: put PCMU (G.711 μ-law) first. Do not use low-bandwidth codecs such as G.729 or G.723 for fax.

Leave T.38 redundancy / packet settings at their defaults for the first test. Change one thing at a time only if troubleshooting later.

Apply / Save, then confirm the HT801 status page says the FXS account is Registered before proceeding.

Do not confuse “dial tone” with registration: The PHONE port can provide dial tone even when the SIP account is not registered. Check the HT801 Status page before testing fax calls.

# 5. Set the HP LaserJet 4345 MFP

On the HP control panel, go to Configure Device → Fax (or run the Fax Setup Wizard). The exact labels can differ slightly by firmware.

| HP setting | Recommended starting value |
| --- | --- |
| Fax header / station name | Your business or personal fax name |
| Fax number / CSID | The 10-digit fax DID |
| Receive mode | Automatic |
| Rings to answer | 2 (dedicated fax line) |
| Dial type | Tone |
| Dial-tone detect | On initially; turn it off only if the HP refuses to dial despite a working HT801 line |
| ECM (error correction) | On initially |
| Fax speed / maximum baud | Start at 9600 if available. Otherwise turn V.34 off to limit to 14.4 kbps for the first test. |

# 6. Test in this order

Check the HT801 status page: the configured FXS account must show Registered.

Send one plain, single-page black-and-white test fax from the HP to a known working fax number. Avoid photo pages at first.

Confirm the outgoing call appears in VoIP.ms call detail records and that the destination receives the page.

Call the DID from an outside phone. It should ring; after two rings the HP should answer with fax tones. Then send it a one-page fax.

Once both directions work, raise the HP fax speed only if you need it. Reliability matters more than maximum speed on an ATA fax line.

# Fast troubleshooting

| Symptom | Check first |
| --- | --- |
| Calling the DID gives fast busy before ringing | VoIP.ms DID activation/routing. Confirm it is active and routed directly to the fax sub-account. |
| HT801 will not register | POP hostname, full sub-account username, SIP password, and router Internet access. Do not use the VoIP.ms portal password. |
| HP says no dial tone | RJ-11 is in HT801 PHONE and HP FAX/LINE. Confirm the HT801 is powered and registered; retry with dial-tone detect off only if needed. |
| Outgoing calls connect but fax fails or retries | Verify T.38 Auto Detect and PCMU first; keep HP speed at 9600/14.4. Then test with ECM off only if the failure persists. |
| Inbound call rings but HP does not answer | HP Receive Mode = Automatic, rings-to-answer = 2, and cable is in the HP FAX/LINE jack. |
| Only some remote faxes fail | That can be the other side’s fax/VoIP setup. Keep a slow-speed one-page test result as your baseline and collect the error report. |

# Completion checklist

☐ DID rings normally from an outside phone

☐ DID routes directly to the VoIP.ms fax sub-account

☐ HT801 FXS account shows Registered

☐ HP is connected to HT801 PHONE → HP FAX/LINE

☐ HP sends a one-page fax successfully

☐ HP receives a one-page fax successfully

☐ Final settings and POP hostname recorded above

Baseline configuration: VoIP.ms + Grandstream HT801 + dedicated HP 4345 MFP fax line. If a menu label differs, use the equivalent setting shown by the HT801/HP firmware rather than changing unrelated advanced options.