Water Pump Replacement Punch List

2010 Jeep Grand Cherokee • 5.7L HEMI V8

Important: Confirm part fitment by VIN. Use the factory-specified HOAT coolant for this model; do not mix coolant types. Work only on a fully cool engine.

# Parts to Buy / Replace

☐  Water pump for the 5.7L HEMI — Confirm fitment by VIN

☐  New water-pump gasket or O-ring — Usually included with a quality pump

☐  Thermostat and thermostat-housing seal — Good preventive replacement while coolant is drained

☐  Correct Jeep/Mopar-compatible HOAT coolant — Enough for refill; use distilled water if mixing concentrate

☐  Serpentine belt — Replace if cracked, glazed, noisy, contaminated, or age is unknown

# Inspect Before Ordering More Parts

☐  Belt tensioner and idler pulleys — Replace if noisy, rough, loose, or wobbling

☐  Upper and lower radiator hoses and clamps — Replace if soft, swollen, cracked, leaking, or original

☐  Radiator cap — Replace if its seal is damaged or it fails a pressure test

☐  Cooling fan operation and radiator condition — Do not replace unless inspection/testing identifies a fault

# Installation / Shop Checklist

☐  Start with a completely cold engine; safely capture and dispose of old coolant

☐  Inspect the old pump for seepage, bearing noise, pulley wobble, or damaged impeller

☐  Clean the pump mating surface carefully; do not gouge the sealing surface

☐  Install the new pump and gasket/O-ring per the service-manual torque sequence/specification

☐  Install thermostat/seal and any approved belt, hose, pulley, or cap replacements

☐  Refill with the correct coolant mixture and bleed all air from the cooling system

# Before Calling It Finished

☐  Run to operating temperature and confirm the cabin heater stays hot

☐  Verify the temperature gauge remains normal and cooling fans cycle normally

☐  Check for leaks around the water pump, thermostat housing, hoses, and radiator

☐  After the engine fully cools, recheck coolant level and top off if needed

☐  Road-test briefly; recheck for leaks, belt noise, overheating, and warning lights

This is a practical planning checklist, not a substitute for the factory service manual or professional diagnosis.